Dynamic library for TCP/IP communication with a laser system

Ready to use for Visual C++ and/or Visual Basic

Revision 4: ScanLinux 3.5.0 and higher necessary to support all commands

Note:  For Visual Basic open the SocketCommVBTest.vbp project to learn how to use 	this DLL. Please note that for Visual Basic the function calls are different, as 	well as the passed function-variables. Open the project to find out how this DLL

can be used within Visual Basic. All functions will be renamed in Visual Basic 	with the Alias statement. However, the functionality of the function calls is the 	same as explained as follows for C/C++.

Files: 	-SocketCommDll.lib: the library, that you have to include for linking

-.\SocketCommDllExport.h: the header file, that is necessary to import and 	use the functions of the dynamic library

-SocketCommDll.dll: the dynamic link library that exports the functions

(must be registered by your Windows system or simply be present in the 	path

of your executable program that uses the functions of this DLL)

- .\SocketCommCPPTest\ SocketCommCPPTest.dsw, ...: a simple Visual C++ 	project that shows you how to use the dynamic link library.

Important:

you have to link your application with the 'socketcommdll.lib' to use the socket communication with the laser system. Additionally, the 'SocketCommDll.dll' should

be either registered or present in your applications executable path.

Headerfile 'SocketCommDllExport.h':

//////////////////////////////////////////////////////////////

//	Status structure: can be filled with MLaser_Status() call.

//

//

//

//////////////////////////////////////////////////////////////

typedef struct{

unsigned long	d_counter;//actual counter of ok-prints

unsigned long	s_counter;//actual counter of nok-prints

unsigned long	n_messageport;//actual external message selection

unsigned char	Start;//0: system is printing 1: system is stopped

unsigned char	request;//(internal variable)

unsigned char	option;//(internal variable)

unsigned char	res;//reserved (0x0:default, 0x1: UMT enabled, 0x4: Batchjob enabled)

unsigned long	t_counter;//total counter of prints

unsigned long	m_copies;//actual nr of copies to be printed

unsigned long err;//alarmcode (upper WORD: codifies lastactive alarm; lower WORD: 0: noalarm, else: alarm active)

unsigned long time;//reserved D-Word (actually used as timeofprint-info)

char  name[8];//actual active filename (max.8 chars)

unsigned long reserved1;	//reserved

unsigned long reserved2;	//reserved

} PStatus;

Description of the important variables:

This is the status structure with the most important status variables of the laser. The variable may be filled with a call to Laser_Status(&p).

d_counter: an internal counter, that counts the "good" prints (prints without errors). The variable can be resetted to zero wtih a call to Laser_CounterReset().

s_counter: an internal counter, that counts the "bad" prints (prints with errors). The variable can be resetted to zero wtih a call to Laser_CounterReset().

t_counter: an internal counter, that counts the total prints. This variable cannot be resetted.

n_messageport: this variable shows the actual BYTE value of the external message selection, when the system is in the mode "external selection".

Start: 0: system is not in the printing mode, 1: system is ready for printing (already printing or waiting for the print signal via photocell or PLC)

err: variable that codifies the internal alarms of the system (devided in upper and lower WORD).

upper WORD (err>>16):

is used to codify the last active alarm (0: no alarm 	is active, <> 0: an 			alarm is active, the value codifies the actual alarm.

lower WORD (err & 0xFFFF):

0:  	no alarm is active

<>0: 	some alarm is active

Note: you should use only the upper WORD to get access to the alarmstatus. The lower WORD is used for some internal functionality.

name: an array of 8 BYTES that contains the name of the actual active printing file, not necessarily ending with '\0'.

The DLL functions:

void MInit(long *p,const char *name,const char *ip,const char *path)

p: pointer to a long variable. The Minit() function fills this variable with a

long number that must be used as a reference for all other function calls with this 	connection.

name: name of the system to be connected to; actually without meaning and 	reserved for future application.

ip: string of the IP of the system to connect to (e.g. "192.168.0.180")

path: string of the default path to be used when copying local files (e.g. ".\\")

NOTE: In all following function calls the value of *p, that was assigned by the Minit() function call, must be passed to the function calls. It is the value of the internally used communication handle.

Up to 24 connections can be opened simultaneously. If the return value (p) is zero, you probably may have opened already 24 connections. You must then call the MFinish() function for one of your open connections.

int MStartClient(long p)

Opens a communication socket to the system. Returns 0 if the communication i 	established; returns <>0 if an error occured. This function must be called once 	after constructing a socketcomm object. Once the function was called 	succesfully, you can send control commands to the laser system.

p:  the value of the communication handle assigned by Minit()

void MFinish(long p)

Shuts down the communication socket. This function should be the last function 	call before leaving the program. It cuts the socket communication and frees 	some internally reserved memory.

p:  the value of the communication handle assigned by Minit()

int	MLaser_Knockout(long p)

Indicates to the connected laser system, that the connection will be shut down. 	This function call is necessary to close carefully the socket connection of the

laser system and hsould be called before the ShutdownClient() function.

p:  the value of the communication handle assigned by Minit()

int 	MGetLastError(long p ,char &txt)

Fills the txt-array of max. 256-bytes. The provided string contains a description

of the error the occured (only communication 	errors). Usually, you should 	not get any communication error, except a timeout 	error when the laser 	system 	is blocked.

p:  the value of the communication handle assigned by Minit()

int MIsConnected(long p)

Returns 0 if the system is not connected and 1 if the system is connected.

p:  the value of the communication handle assigned by Minit()

unsigned short MGetVersion(long p)

Returns the internal version number of remote serverprogram.

p:  the value of the communication handle assigned by Minit()

int MSetTimeout(long p ,int  timeout)

Sets communication timeout variable. The value is passed in units of 	milliseconds. The default value (if this function is not called) is 2000 ms.

If the time that passes since start of a command sent to the laser and until 	receiving an answer from the laser is greater than the timeout, a timeout error

occures and the corresponding function call  returns with an error.

p:  the value of the communication handle assigned by Minit()

int MLaser_Start(long p ,const char *filename,int nr)

Indicates the system to enter into the printing mode with the actual file set to 	<filename>. The <filename> must be passed without extension.

int nr:  number of prints to be done (if nr is equal to 1, a testprint will be done 	immediately; if nr is equal to 0, the system enters into an eternal printing mode 	until the stop signal is sent, or until some internal error occures.)

p:  the value of the communication handle assigned by Minit()

int		MLaser_Stop(long p ,long timeout)

Send the stop signal to the system. The laser stops immediately printing and 	leaves the printing mode. An optional timeout value can be passed to the 	function call (if timeout is equal to zero, the default timeout is assumed).

p:  the value of the communication handle assigned by Minit()

int		MLaser_Reload(long p)

Commands the system to reload the actual printing file. This call is usually used 	after having sent an updated printing file to the laser.

p:  the value of the communication handle assigned by Minit()

int		MLaser_CounterReset(long p)

Resets the internal d_counter and s_counter of the laser's internal status 	structure. Recommended to use each time before calling the Laser_Start() 	function, so you can easiely control the number of prints since having entered 	into the printing mode.

p:  the value of the communication handle assigned by Minit()

int		MLaser_Settime(long p)

Sets the actual system time in the remote lasersystem. The local operation 	system time is used as the reference time.

p:  the value of the communication handle assigned by Minit()

int     MLaser_Delete(long p ,const char *name)

The remote file <name> will be deleted inside the laser. Use carefully !

p:  the value of the communication handle assigned by Minit()

int     MLaser_SetDefault(long p ,const char *name)

Changes the actual printing file to <name>. No extension must be set in 	<name>. The system will load the selected file <name> for printing.

p:  the value of the communication handle assigned by Minit()

int	MLaser_CopyFile(long p ,const char * sourcefile, const char * path, unsigned char option)

Function to copy files to/from the laser system. Usually is reserved for further 	and more complex applications.

sourcefile: local filename to be sent to the laser (with extension)

path: directory ending with "\\" of location of the source file.				option: copy to/from laser

0: copy to RAMDisk of laser

1: copy to Harddisk of Laser

2: copy from Harddisk of Laser

4: copy from RAMdisk of Laser

p:  the value of the communication handle assigned by Minit()

int	MLaser_Status(long p ,PStatus &status)

Fills the status variable <status> with the actual values of the system. Function is 	used to check the actual status of the laser system.

p:  the value of the communication handle assigned by Minit()

int	MLaser_Mode(long p ,unsigned char &mode)

Sets the actual mode of the laser. The laser can work in 4 different modes:

0 = static mode:

1= dynamic mode

2= dynamic distance mode

3= dynamic-static mode

If you want to request the actual mode, you have to set the <mode> variable to 8. 	If the function returns 0, the <mode> variable contains the actual mode.

Note: this function is used only for some special applications where the mode 	must be switched between static and dynamic.

p:  the value of the communication handle assigned by Minit()

int	MLaser_Usermessage(long p ,unsigned char field,const char *text)

Sets the string of an internal usermessage field to <text>.

unsigned char field:  the field of the internal usermessage, that will be 			overwritten.

const char *text: a pointer to a string array of max. 127 chars.

p:  the value of the communication handle assigned by Minit()

int	MLaser_FastUsermessage(long p ,unsigned char field,const char *text)

Sets the string of an internal usermessage field to <text>.

unsigned char field:  the field of the internal usermessage, that will be 			overwritten.

const char *text: a pointer to a string array of max. 127 chars.

p:  the value of the communication handle assigned by Minit()

This command is available for ScanLinux 3.6.0 and higher. It is slightly faster than the “MLaser_Usermessage” command.

int	MLaser_SetGlobalCounter (long p ,unsigned char field,const char *counter)

Sets the value of a global internal counter

unsigned char field:  the field of the internal counter

const char *counter: a pointer to a ‘\0’ terminated string array containing the counter value.

p:  the value of the communication handle assigned by Minit()

int	MLaser_GetGlobalCounter (long p ,unsigned char field, char &counter)

Gets the value of a global internal counter

unsigned char field:  the field of the internal counter

char &counter: a reference to a ‘\0’ terminated string array to where the countervalue should be copied. Be sure to reserve an array that will be big enough (e.g. 48 bytes)

p:  the value of the communication handle assigned by Minit()

int	MLaser_Offset (long p, long &dx, long &dy, long relative,long format /*=0*/,long reset /*=0*/)

Sets a global offset (dx,dy) for all messages in static printing.

dx: Offset in x

dy: Offset in y

relative: if set to 0: offset is absolute

if set to 1: offset is relative	(relative to any previous offset)

format: (optional for ScanLinux3.6.0 and higher)

0: units are ideal units (100 000 units = Scanfield)

1: units are microns

2: units are 0,1 mm

reset: (optional for ScanLinux3.6.0 and higher)

0: offset valid for all prints

< > 0: offset will be used only for the next print. Any subsequent prints

will be printed with zero offset if no new offset is sent to the laser.

p:  the value of the communication handle assigned by Minit()

ScanLinux3.5.0 (and higher):

Note: The offset is valid only for static printing. If the offset is absolute, all messages will be shifted by the (dx,dy)-value sent to the laser. To reset the offset, you have to send (0,0) to the laser.

If you send a relative offset, the value you have sent will be added to the actual internal offset.

In both cases, absolute or relative, the function-call fills both variables (dx, dy	) with the actual internal global offset, so you can monitor the actual offset.

Units: (dx,dy)  have integer values. A value of +50 000 would shift the message by half of the total scanfield. So, the units are scanfield dependent.

No control check will be done when the shift is applied, so the user must assure to stay within the total scanfield.

ScanLinux3.6.0 (and higher):

The offset is valid for static and dynamic printing.

How to use the DLL functions ???

Before using any command to communicate with the laser system you have to call the MInit(...) function.

This will open a communication socket with the parameters that you passed to the function. The MInit() call will initialize a value for the communication handle (long *p) that must be passed to all following function calls.

This is necessary to identify a specific connection to a lasersystem, e.g. when you work with multiple systems.

After this you must connect to the laser with the function call:

err = MStartClient();

If the connection is established corretly, the function returns 0 else it return <> 0 	and you can get a description of the error by a call to MGetLastError().

Remember, that you can set the timeout any time with MSetTimeout(value).

If the MStartClient() function call returns 0, you will have a connection to the laser system and from now on you can communicate with the system. That means, you can make any function call that starts with MLaser_..... (e.g. MLaser_Status(), MLaser_Start(), .....).

When you want to finish a connection to the laser you should first call the MLaser_Knockout() function to tell the laser system that the communciation will be shut down. Then you must call the MFinish() function to close correctly the sockets..

Note: see the SocketCommCPPTest.dsw project.

ALARM codes of upper WORD of status.err:

0x02: Laser is OFF (interlock open)

0x03: shutter is closed

0x04: DC Power fails

0x05: Overtemp of amplifier

0x06: Q-switch overtemp

0x07: high reverse power

0x08: low forward power

0x09: YAG RS232 error

0x0A: belt stopped

0x0B: program check not ok

0x0C: wrong figure-types in file

0x0D: no memory available

0x10: file not found

0x11: waterflow fails

0x12: interlock

0x14: laserfail

0x15: invalid font (or not existing)

0x16: overtemp.

0x25: shutter closed

0x26: laser not ready

:	0x30: overspeed

0x31: harddisk full

0x32: file not allowed to print

0x33: barcode creation failed (wrong parameters)

0x34: no barcode license

0x35: no barcode library (wrong version)